/*

* ESP8266 Hydroponic Automation Sensor Module

* Author: Varad Chaskar

* Description: This code is part of a hydroponic automation project and is designed for the ESP8266 microcontroller.

*              It gathers environmental data using sensors, including temperature, humidity, light intensity, and water level.

*              The collected data is transmitted to the ThingSpeak cloud platform for real-time monitoring and analysis.

* Date: January 28, 2024

*/

#include <WiFiManager.h> 				// Library for simplified WiFi connection handling

#include <ESP8266WiFi.h> 				// ESP8266 WiFi library

#include <DHT.h> 					// DHT sensor library for interfacing with DHT11 sensor

#include "ThingSpeak.h" 				// Library for seamless communication with ThingSpeak IoT platform

WiFiClient client; 					// WiFi client for ThingSpeak communication

unsigned long myChannelNumber = 2384399;              		// Channel number on ThingSpeak cloud

const char *myWriteAPIKey = "ELW2NF5Q83OGB39G";        	// Write API key for ThingSpeak

const char *myCounterReadAPIKey = "3D8NH4JCI0EDYMIU"; 	 // Read API key for ThingSpeak

DHT dht; 					// Initialize DHT sensor

const int DHTPIN = D5; 				// Pin for DHT sensor

void setup() {

pinMode(LED_BUILTIN, OUTPUT); 		// Set built-in LED pin as output

Serial.begin(115200);			 	// Initialize serial communication for debugging

WiFiManager wm; 				// Create WiFiManager instance for easy WiFi connection handling

bool res;

res = wm.autoConnect("ASAA"); 			// Attempt to connect to WiFi using saved credentials

if(!res) {

Serial.println("Failed to connect");

digitalWrite(LED_BUILTIN, HIGH); 		// Turn on built-in LED to indicate connection failure

delay(1000);

digitalWrite(LED_BUILTIN, LOW); 		// Turn off built-in LED

delay(1000);

digitalWrite(LED_BUILTIN, HIGH); 		// Turn on built-in LED

ESP.restart(); 					// Restart the ESP8266

}

else {

ThingSpeak.begin(client);			// Initialize ThingSpeak communication

digitalWrite(LED_BUILTIN, LOW); 		// Turn off built-in LED to indicate successful connection

Serial.println("Connected to WiFi");

dht.setup(DHTPIN); 				// Initialize DHT sensor

}

}

void loop() {

digitalWrite(LED_BUILTIN, LOW); 		// Turn off built-in LED

// Read DHT sensor data

float temperature = dht.getTemperature(); 		// Read temperature from DHT sensor

float humidity = dht.getHumidity(); 			// Read humidity from DHT sensor

// Read LDR analog input

int ldrValue = analogRead(A0);			// Read analog input from LDR

// Read water level (0 or 1) from ThingSpeak

int waterLevel = digitalRead(D0);

// Send data to ThingSpeak

ThingSpeak.writeField(myChannelNumber, 1, temperature, myWriteAPIKey); 	// Send temperature data to ThingSpeak

ThingSpeak.writeField(myChannelNumber, 2, humidity, myWriteAPIKey); 		// Send humidity data to ThingSpeak

ThingSpeak.writeField(myChannelNumber, 3, ldrValue, myWriteAPIKey); 		// Send LDR value to ThingSpeak

ThingSpeak.writeField(myChannelNumber, 4, waterLevel, myWriteAPIKey); 	// Send water level data to ThingSpeak

}

/*

* ESP8266 Hydroponic Automation Sensor Module

* Author: Varad Chaskar

* Description: This code is part of a hydroponic automation project and is designed for the ESP8266 microcontroller.

*              It gathers environmental data using sensors, including temperature, humidity, light intensity, and water level.

*              The collected data is transmitted to the ThingSpeak cloud platform for real-time monitoring and analysis.

* Date: 28 Jan. 2024

*/

#include <WiFiManager.h> // Library for simplified WiFi connection handling

#include <ESP8266WiFi.h> // ESP8266 WiFi library

#include <DHT.h> // DHT sensor library for interfacing with DHT11 sensor

#include "ThingSpeak.h" // Library for seamless communication with ThingSpeak IoT platform

WiFiClient client; // WiFi client for ThingSpeak communication

unsigned long myChannelNumber = 2384399;               // Channel number on ThingSpeak cloud

const char *myWriteAPIKey = "ELW2NF5Q83OGB39G";        // Write API key for ThingSpeak

const char *myCounterReadAPIKey = "3D8NH4JCI0EDYMIU";  // Read API key for ThingSpeak

DHT dht; // Initialize DHT sensor

const int DHTPIN = D5; // Pin for DHT sensor

void setup() {

pinMode(LED_BUILTIN, OUTPUT); // Set built-in LED pin as output

Serial.begin(115200); // Initialize serial communication for debugging

WiFiManager wm; // Create WiFiManager instance for easy WiFi connection handling

bool res;

res = wm.autoConnect("ASAA"); // Attempt to connect to WiFi using saved credentials

if(!res) {

Serial.println("Failed to connect");

digitalWrite(LED_BUILTIN, HIGH); // Turn on built-in LED to indicate connection failure

delay(1000);

digitalWrite(LED_BUILTIN, LOW); // Turn off built-in LED

delay(1000);

digitalWrite(LED_BUILTIN, HIGH); // Turn on built-in LED

ESP.restart(); // Restart the ESP8266

}

else {

ThingSpeak.begin(client); // Initialize ThingSpeak communication

digitalWrite(LED_BUILTIN, LOW); // Turn off built-in LED to indicate successful connection

Serial.println("Connected to WiFi");

dht.setup(DHTPIN); // Initialize DHT sensor

}

}

void loop() {

digitalWrite(LED_BUILTIN, LOW); // Turn off built-in LED

// Read DHT sensor data

float temperature = dht.getTemperature(); // Read temperature from DHT sensor

float humidity = dht.getHumidity(); // Read humidity from DHT sensor

// Read LDR analog input

int ldrValue = analogRead(A0); // Read analog input from LDR

// Read water level (0 or 1) from ThingSpeak

int waterLevel = ThingSpeak.readLongField(myChannelNumber, 4, myCounterReadAPIKey);

// Send data to ThingSpeak

ThingSpeak.writeField(myChannelNumber, 1, temperature, myWriteAPIKey); // Send temperature data to ThingSpeak

ThingSpeak.writeField(myChannelNumber, 2, humidity, myWriteAPIKey); // Send humidity data to ThingSpeak

ThingSpeak.writeField(myChannelNumber, 3, ldrValue, myWriteAPIKey); // Send LDR value to ThingSpeak

ThingSpeak.writeField(myChannelNumber, 4, waterLevel, myWriteAPIKey); // Send water level data to ThingSpeak

}