/*

* File: ActuatorModule.ino

* Author: Varad Chaskar

* Description: Code for Actuator Module using ESP8266 and ThingSpeak.

* Date: January 28, 2024

*/

#include <WiFiManager.h>  			// Include the WiFiManager library for easy WiFi configuration

#include <ESP8266WiFi.h>  			// Include the ESP8266WiFi library for WiFi functionality

#include "ThingSpeak.h"   			// Include the ThingSpeak library for communication with ThingSpeak cloud

WiFiClient client;  				// Create a WiFi client object for connecting to the internet

unsigned long myChannelNumber = 2384399;               		// Channel number taken from cloud

const char *myWriteAPIKey = "ELW2NF5Q83OGB39G";        	// Write API key of cloud

const char *myCounterReadAPIKey = "3D8NH4JCI0EDYMIU";  	// Read API key from cloud

bool LED = false;      			// Variable to track the state of LED

bool LEDValue = false; 			// Variable to track the state of LEDValue

void setup() {

pinMode(LED_BUILTIN, OUTPUT);  		// Set the built-in LED pin as an output

pinMode(0, OUTPUT);           		// Set pin 0 as an output

pinMode(D7, OUTPUT);           		// Set pin D7 as an output

pinMode(D8, OUTPUT);           		// Set pin D8 as an output

pinMode(4, OUTPUT);            		// Set pin 4 as an output

pinMode(5, OUTPUT);            		// Set pin 5 as an output

Serial.begin(115200);  			// Start serial communication with baud rate 115200

WiFiManager wm;  			// Create a WiFiManager instance

bool res;

res = wm.autoConnect("ASAA");  		// Attempt to connect to WiFi with SSID "ASAA"

if (!res) {

// Failed to connect to WiFi

Serial.println("Failed to connect");

digitalWrite(LED_BUILTIN, HIGH);  	// Turn on the built-in LED

delay(1000);

digitalWrite(LED_BUILTIN, LOW);  	// Turn off the built-in LED

delay(1000);

digitalWrite(LED_BUILTIN, HIGH);  	// Turn on the built-in LED

ESP.restart();  				// Restart the ESP8266

} else {

// Successfully connected to WiFi

ThingSpeak.begin(client);       		// Initialize ThingSpeak with the WiFi client

digitalWrite(LED_BUILTIN, LOW);  	// Turn off the built-in LED

Serial.println("Connected... Yeey :)");  	// Print message to serial monitor

}

}

void loop() {

digitalWrite(LED_BUILTIN, LOW);  		// Turn off the built-in LED

int A = ThingSpeak.readLongField(myChannelNumber, 5, myCounterReadAPIKey);  // Read data from ThingSpeak channel field 5

analogWrite(5, A); 	 		// Set the analog output on pin 5 based on the read value

int B = ThingSpeak.readLongField(myChannelNumber, 6, myCounterReadAPIKey);  // Read data from ThingSpeak channel field 6

analogWrite(4, B);  			// Set the analog output on pin 4 based on the read value

int C = ThingSpeak.readLongField(myChannelNumber, 7, myCounterReadAPIKey);  // Read data from ThingSpeak channel field 7

analogWrite(0, C);  			// Set the analog output on pin 0 based on the read value

int D = ThingSpeak.readLongField(myChannelNumber, 8, myCounterReadAPIKey);  // Read data from ThingSpeak channel field 8

if (D == 100 && LED == false) {

// Toggle LED and indicate with built-in LED

digitalWrite(D7, HIGH);

delay(1000);

digitalWrite(D7, LOW);

LED = true;

digitalWrite(LED_BUILTIN, HIGH);

delay(500);

digitalWrite(LED_BUILTIN, LOW);

}

if (D == 0 && LED == true) {

// Toggle LED and indicate with built-in LED

digitalWrite(D7, HIGH);

delay(1000);

digitalWrite(D7, LOW);

digitalWrite(LED_BUILTIN, HIGH);

delay(500);

digitalWrite(LED_BUILTIN, LOW);

LED = false;

}

if (D == 200 && LEDValue == false) {

// Toggle LEDValue and indicate with built-in LED

digitalWrite(D8, HIGH);

delay(1000);

digitalWrite(D8, LOW);

LEDValue = true;

digitalWrite(LED_BUILTIN, HIGH);

delay(500);

digitalWrite(LED_BUILTIN, LOW);

}

if (D == 300 && LEDValue == true) {

// Toggle LEDValue and indicate with built-in LED

digitalWrite(D8, HIGH);

delay(1000);

digitalWrite(D8, LOW);

LEDValue = false;

digitalWrite(LED_BUILTIN, HIGH);

delay(500);

digitalWrite(LED_BUILTIN, LOW);

}

}